TECHNICAL USER MANUAL

# RealHand ROS2 SDK

Installation, Integration, Operation, and API Reference

Table of Contents

1. Project Overview	3

2. Software Architecture	4

3. Repository Structure	5

4. Dependencies	5

5. Installation	6

6. Workspace Setup	6

7. Build Instructions	7

8. Running the SDK	8

9. Launch Files	9

10. ROS2 Nodes	10

11. ROS2 Topics	11

12. ROS2 Parameters	12

13. Message Types	13

14. API Usage	14

15. Common Workflows	15

16. Troubleshooting	16

17. Frequently Asked Questions (FAQ)	17

18. Appendix	18

# 1. Project Overview

The RealHand ROS2 SDK’s nodes construct hand and arm objects from the separately installed realhand Python SDK, adapt their methods to ROS operations, and expose commands and feedback through sensor_msgs/msg/JointState and std_msgs/msg/String.

The following model identifiers are used in this document:

| Device family | Model values used in this document | Command width |
| --- | --- | --- |
| Dexterous hand | L6, O6 | 6 values |
| Dexterous hand | L20 | 16 values |
| Arm | P7, A7lite | 7 values |

## 1.1 Runtime capability boundaries

Repository-verifiable consequences:

Hand metadata confirms command widths and joint order: L6 and O6 use six values, while L20 uses sixteen values.

The touch publisher receives the same force_sensor payload as the force_sensor publisher.

The delivered user-facing capabilities are:

single-hand and dual-hand ROS 2 backends;

typed sensor_msgs/msg/JointState command and state topics;

# 2. Software Architecture

The package uses a layered design:

## 2.1 Layer responsibilities

| Layer | Implementation | Responsibility |
| --- | --- | --- |
| Model metadata | hand/model.py, arm/model.py | Normalizes model names, defines joint order, and lazily resolves external SDK classes. |
| Adapter | hand/adapter.py, arm/adapter.py | Converts Python SDK objects and sensor records into plain lists/dictionaries and wraps device operations. |
| ROS bridge | hand/node.py, arm/node.py | Declares parameters, creates topics and timers, translates messages, and owns adapter lifetime. |
| GUI ROS facade | gui/ros_client.py | Publishes commands and subscribes to selected feedback topics for the active side. |

## 2.2 Threading and concurrency

ROS callbacks and timers run under the default rclpy.spin() executor used by each backend.

The GUI integrates ROS processing into Qt with a 20 ms timer that calls rclpy.spin_once(..., timeout_sec=0.0).

# 3. Repository Structure

The current local repository layout is summarized below.

| Path | Current contents / purpose |
| --- | --- |
| launch/ | arm.launch.py, dual_hand.launch.py, gui.launch.py, and hand.launch.py. |
| resource/ | ament package resource marker. |
| src/realhand_ros2/ | hand/, arm/, gui/, py.typed, and package initializer. |
| test/ | Model metadata and joint-count unit test. |
| Repository root | .gitignore, README.md, package.xml, pyproject.toml, run_gui.sh, setup.cfg, and setup.py. |

# 4. Dependencies

## 4.1 ROS package dependencies

Declared runtime dependencies in package.xml are:

| Dependency | Use |
| --- | --- |
| rclpy | Nodes, publishers, subscribers, parameters, timers, and spinning. |
| sensor_msgs | sensor_msgs/msg/JointState. |
| std_msgs | std_msgs/msg/String for JSON payloads. |
| python3-pyqt5 | Desktop GUI. |

Test dependencies are ament_copyright, ament_flake8, ament_pep257, and python3-pytest.

## 4.2 Python dependencies

setup.py specifies setuptools, realhand, and PyQt5.

Install a compatible RealHand Python SDK into the same Python environment used by ROS 2.

# 5. Installation

## 5.1 Manual installation

sudo apt update

sudo apt install -y \

git \

iproute2 \

can-utils \

python3-pip \

python3-colcon-common-extensions \

python3-pyqt5

Source ROS 2 and install a compatible RealHand Python SDK into the same Python environment used by ROS 2.

# 6. Workspace Setup

Use a conventional ROS 2 workspace layout:

mkdir -p ~/realhand_ros2_ws/src
cd ~/realhand_ros2_ws/src
git clone https://github.com/RealHand-Robotics/realhand-ros2-sdk realhand-ros2-sdk
cd ..

If working from an existing local checkout, copy only source-controlled content. Do not carry old build/, install/, log/, .pytest_cache/, __pycache__/, or *.egg-info/ artifacts into the new workspace.

Every new terminal must source the ROS distribution and, after building, the workspace overlay:

source /opt/ros/humble/setup.bash
source ~/realhand_ros2_ws/install/setup.bash

Optional shell convenience:

echo 'source /opt/ros/humble/setup.bash' >> ~/.bashrc
echo 'source ~/realhand_ros2_ws/install/setup.bash' >> ~/.bashrc

# 7. Build Instructions

From the workspace root:

source /opt/ros/humble/setup.bash
cd ~/realhand_ros2_ws
colcon build --symlink-install --packages-select realhand_ros2
source install/setup.bash

Validate the installation:

ros2 pkg prefix realhand_ros2
ros2 pkg executables realhand_ros2

setup.py defines these executable names:

realhand_ros2 realhand_arm_node
realhand_ros2 realhand_gui
realhand_ros2 realhand_hand_node

## 7.1 Run package tests

cd ~/realhand_ros2_ws
colcon test --packages-select realhand_ros2
colcon test-result --verbose

The repository's only direct unit test verifies hand and arm model joint counts. Hardware communication, topic behavior, launch startup, and GUI operation are not covered by the included automated test.

## 7.2 Clean rebuild

When stale generated metadata or an old copied checkout causes discovery problems, remove only the generated workspace directories from the workspace root, then rebuild:

cd ~/realhand_ros2_ws
rm -rf build install log
source /opt/ros/humble/setup.bash
colcon build --symlink-install --packages-select realhand_ros2
source install/setup.bash

# 8. Running the SDK

## 8.1 Prepare SocketCAN

HandNode defaults to can0/socketcan. The A7lite arm model also defaults to can0/socketcan. Backend nodes pass the selected interface values to adapter constructors and do not execute ip link configuration commands.

sudo ip link set can0 down
sudo ip link set can0 type can bitrate 1000000
sudo ip link set can0 up
ip -details link show can0

Useful diagnostics:

ip link show
candump can0

## 8.2 Start a hand backend

source /opt/ros/humble/setup.bash
source ~/realhand_ros2_ws/install/setup.bash
ros2 launch realhand_ros2 hand.launch.py \
  model:=L6 side:=left interface_name:=can0

## 8.3 Start two hands

ros2 launch realhand_ros2 dual_hand.launch.py \
  model:=L6 left_interface:=can0 right_interface:=can1

This launches two realhand_hand_node processes named realhand_left_hand_node and realhand_right_hand_node. Both use the same model argument but different side and interface values.

## 8.4 Start the GUI

ros2 launch realhand_ros2 gui.launch.py

run_gui.sh verifies /opt/ros/$ROS_DISTRO/setup.bash and the repository-local install/setup.bash, sources both, and executes the GUI launch file. It defaults to Humble.

The GUI is initially a ROS client. Use Start Backend on the applicable tab to launch a matching GUI-owned backend, or run the backend separately before opening the GUI. Stop Backend stops the GUI-owned backend process.

## 8.5 Confirm the ROS graph

ros2 node list
ros2 topic list | grep realhand
ros2 node info /realhand_hand_node

# 9. Launch Files

## 9.1 hand.launch.py

Launches one realhand_hand_node named realhand_hand_node with console output.

| Argument | Default | Required | Meaning |
| --- | --- | --- | --- |
| model | L6 | No | Model value passed to HandNode. |
| side | left | No | Topic and SDK side selector: left or right. |
| interface_name | can0 | No | CAN interface passed to the external SDK. |
| interface_type | socketcan | No | External SDK interface type. |

## 9.2 dual_hand.launch.py

Launches two hand backends. Both use model and interface_type from common arguments.

| Argument | Default | Required | Meaning |
| --- | --- | --- | --- |
| model | L6 | No | Model applied to both hands. Mixed models are not supported by this launch file. |
| left_interface | can0 | No | Interface for the left hand. |
| right_interface | can1 | No | Interface for the right hand. |
| interface_type | socketcan | No | Interface type for both devices. |

The node names are /realhand_left_hand_node and /realhand_right_hand_node. Topic namespaces are generated from each node's side parameter.

## 9.3 arm.launch.py

Launches one realhand_arm_node named realhand_arm_node.

| Argument | Default | Required | Meaning |
| --- | --- | --- | --- |
| model | P7 | No | Arm product model: P7 or A7lite. |
| side | left | No | left or right; used by SDK construction and default topic names. |
| interface_name | can0 | No | Arm interface passed to the node. |
| interface_type | socketcan | No | Arm interface type passed to the node. |
| world_frame | urdf | No | Passed to the external arm constructor. This node does not publish TF. |

## 9.4 gui.launch.py

Launches the realhand_gui executable as node name /realhand_gui.

# 10. ROS2 Nodes

## 10.1 Hand bridge node

| Item | Description |
| --- | --- |
| Purpose | Exposes one supported RealHand hand through typed command/state and JSON control/sensor topics. |
| Executable | realhand_hand_node |
| Default node name | /realhand_hand_node |
| Input topics | /{prefix}/command (JointState), /{prefix}/command_json (String). |
| Output topics | State, snapshot, angle, speed, torque, acceleration, temperature, current, touch, force sensor, and fault topics. |
| Parameters | Model, side, interface, connection, state rate, field fill, and 16 topic-name parameters. |
| Typical use | Publish position, speed, torque, and clear-fault commands and subscribe to feedback. |

The typed hand command mapping is: position -> set_angles, velocity -> set_speeds, effort -> set_torques.

Hand JSON processing order is: the optional action field first, followed by angles, speed, and torques, in that order.

All output publishers listed above are created for every HandNode instance.

## 10.2 Arm bridge node

| Item | Description |
| --- | --- |
| Purpose | Exposes one arm through ROS 2 topics. |
| Executable | realhand_arm_node |
| Default node name | /realhand_arm_node |
| Input topics | /{prefix}/joint_command (JointState), /{prefix}/command_json (String). |
| Output topics | Joint state and pose. |
| Parameters | Model, side, interface, world frame, TCP offset, connection, state rate, and four topic overrides. |
| Typical use | Enable/home an arm with JSON, issue seven-element joint commands, and monitor typed and JSON state outputs. |

The arm typed command order is position, velocity, effort: position -> move_j(..., blocking=False), velocity -> set_velocities(), effort -> set_accelerations().

## 10.3 GUI ROS client node

| Item | Description |
| --- | --- |
| Purpose | Provides the ROS communication facade used by the PyQt desktop application. |
| Executable | realhand_gui |
| Node name | /realhand_gui |
| Hand inputs | State, snapshot, temperature, current, and touch for the selected side. |
| Hand outputs | Hand command and command_json for the selected side. |
| Arm inputs | Arm joint_state and pose for the selected side. |
| Arm outputs | Arm joint_command and command_json for the selected side. |
| Typical use | Interactive operation, sensor display, presets, auto-grab, slip detection, and starting/stopping a GUI-owned backend process. |

# 11. ROS2 Topics

All create_publisher() and create_subscription() calls in the backend and GUI nodes pass 10 as the queue-depth argument.

In the tables below, {side} is left or right. Backend topic names can be replaced with startup parameters when a node is run directly. The GUI client uses hard-coded default topic names and does not discover or follow backend topic overrides.

## 11.1 Hand command topics

| Topic | Type | Direction relative to backend | Description |
| --- | --- | --- | --- |
| /realhand/{side}/hand/command | sensor_msgs/msg/JointState | Subscribe | Typed hand commands: position sets angles, velocity sets speeds, and effort sets torques. |
| /realhand/{side}/hand/command_json | std_msgs/msg/String | Subscribe | JSON hand commands containing angles, speeds, torques, and the clear_faults (L6 and L20 only).action. |

Typed field mapping:

| JointState field | Hand meaning | Backend handling |
| --- | --- | --- |
| name | Informational only; the backend does not reorder by name. | JointState.name is not read by the command callback. |
| position | Angle/position targets sent to HandAdapter.set_angles(). | No clamping or vector-length validation in the ROS node. |
| velocity | Speed targets sent to HandAdapter.set_speeds(). | No clamping or vector-length validation in the ROS node. |
| effort | Torque targets sent to HandAdapter.set_torques(). | No clamping or vector-length validation in the ROS node. |

The hand ROS node does not clamp command values or validate command vector lengths.

## 11.2 Hand feedback topics

| Topic | Type | Typical source and behavior |
| --- | --- | --- |
| /realhand/{side}/hand/state | sensor_msgs/msg/JointState | Periodic cached angle/speed/torque snapshot. Missing vectors are zero-filled by default. |
| /realhand/{side}/hand/snapshot | std_msgs/msg/String | Periodic JSON conversion of get_snapshot(). |
| /realhand/{side}/hand/angle | std_msgs/msg/String | JSON angle snapshot and/or AngleEvent.data. |
| /realhand/{side}/hand/speed | std_msgs/msg/String | JSON speed snapshot and/or SpeedEvent.data. |
| /realhand/{side}/hand/torque | std_msgs/msg/String | JSON torque snapshot and/or TorqueEvent.data. |
| /realhand/{side}/hand/acceleration | std_msgs/msg/String | JSON acceleration snapshot and/or AccelerationEvent.data, if supported. |
| /realhand/{side}/hand/temperature | std_msgs/msg/String | JSON temperature snapshot and/or TemperatureEvent.data. |
| /realhand/{side}/hand/current | std_msgs/msg/String | JSON current snapshot and/or CurrentEvent.data, if supported. |
| /realhand/{side}/hand/force_sensor | std_msgs/msg/String | JSON force-sensor data. |
| /realhand/{side}/hand/touch | std_msgs/msg/String | Alias publication containing the same force-sensor payload. |
| /realhand/{side}/hand/fault | std_msgs/msg/String | JSON fault snapshot and/or FaultEvent.data, if supported. |

The node creates all listed publishers for every model. An optional sensor publisher emits a message only when the corresponding snapshot or event payload is not None.

## 11.3 Arm topics

| Topic | Type | Direction relative to backend | Description |
| --- | --- | --- | --- |
| /realhand/{side}/arm/joint_command | sensor_msgs/msg/JointState | Subscribe | Joint positions, velocities, and accelerations. |
| /realhand/{side}/arm/command_json | std_msgs/msg/String | Subscribe | Arm actions and direct command arrays. |
| /realhand/{side}/arm/joint_state | sensor_msgs/msg/JointState | Publish | Seven joint names with positions, velocities, and torque readings in effort. |
| /realhand/{side}/arm/pose | std_msgs/msg/String | Publish | JSON Cartesian pose with x, y, z, rx, ry, and rz. |

For arm command JointState, effort carries acceleration commands. For arm feedback JointState, effort carries values returned by the SDK's get_torques(). This asymmetry is important for application design.

# 12. ROS2 Parameters

Parameters are read during node construction. The source does not install parameter-change callbacks, so changing a parameter after startup does not reconfigure the running backend.

## 12.1 Hand node parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| model | string | L6 | Passed to get_hand_model_spec(). |
| side | string | left | Side used by SDK constructor and default topics. |
| interface_name | string | can0 | Device interface. |
| interface_type | string | socketcan | External SDK interface type. |
| connect_on_start | bool | true | If false, topics and timers exist but hardware commands are ignored and no state is published. |
| state_rate_hz | double | 30.0 | Periodic state and snapshot publication timer; zero selects the 30 Hz fallback. |
| fill_missing_fields | bool | true | Zero-fill absent position/velocity/effort vectors to the model joint count. If false, absent vectors are empty. |
| command_topic | string | empty | Empty resolves to /realhand/{side}/hand/command. |
| json_command_topic | string | empty | Empty resolves to /realhand/{side}/hand/command_json. |
| state_topic | string | empty | Empty resolves to /realhand/{side}/hand/state. |
| snapshot_topic | string | empty | Empty resolves to /realhand/{side}/hand/snapshot. |
| angle_topic | string | empty | Empty resolves to /realhand/{side}/hand/angle. |
| speed_topic | string | empty | Empty resolves to /realhand/{side}/hand/speed. |
| torque_topic | string | empty | Empty resolves to /realhand/{side}/hand/torque. |
| acceleration_topic | string | empty | Empty resolves to /realhand/{side}/hand/acceleration. |
| temperature_topic | string | empty | Empty resolves to /realhand/{side}/hand/temperature. |
| current_topic | string | empty | Empty resolves to /realhand/{side}/hand/current. |
| touch_topic | string | empty | Empty resolves to /realhand/{side}/hand/touch. |
| force_sensor_topic | string | empty | Empty resolves to /realhand/{side}/hand/force_sensor. |
| fault_topic | string | empty | Empty resolves to /realhand/{side}/hand/fault. |

Run directly with parameters not exposed by the launch file:

ros2 run realhand_ros2 realhand_hand_node --ros-args \
  -p model:=L20 \
  -p side:=right \
  -p interface_name:=can1 \
  -p state_rate_hz:=20.0 \
  -p fill_missing_fields:=false

## 12.2 Arm node parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| model | string | P7 | Arm product model: P7 or A7lite. |
| side | string | left | Side used by SDK and default topic names. |
| interface_name | string | can0 | Arm interface passed to the adapter. |
| interface_type | string | socketcan | Arm interface type passed to the adapter. |
| world_frame | string | urdf | Passed to external arm constructor. |
| tcp_offset | double array | [0.0, 0.0, 0.0] | Passed to the external arm constructor. |
| connect_on_start | bool | true | If false, hardware is not constructed and commands are ignored. |
| state_rate_hz | double | 50.0 | Joint/pose timer rate; zero falls back to 50 Hz. |
| joint_command_topic | string | empty | Empty resolves to /realhand/{side}/arm/joint_command. |
| json_command_topic | string | empty | Empty resolves to /realhand/{side}/arm/command_json. |
| joint_state_topic | string | empty | Empty resolves to /realhand/{side}/arm/joint_state. |
| pose_topic | string | empty | Empty resolves to /realhand/{side}/arm/pose. |

## 12.3 Inspect parameters

ros2 param list /realhand_hand_node
ros2 param get /realhand_hand_node model
ros2 param dump /realhand_hand_node

# 13. Message Types

## 13.1 sensor_msgs/msg/JointState

The package uses the standard message without custom extensions.

### Selected hand joint ordering

| Model | Ordered names |
| --- | --- |
| L6, O6 | thumb_flex, thumb_abd, index, middle, ring, pinky |
| L20 | thumb_abd, thumb_yaw, thumb_root1, thumb_tip, index_abd, index_root1, index_tip, middle_abd, middle_root1, middle_tip, ring_abd, ring_root1, ring_tip, pinky_abd, pinky_root1, pinky_tip |

### Arm joint ordering

The supported arm product names in this guide are P7 and A7lite. Both use seven ordered joint names: joint_1 through joint_7.

joint_1, joint_2, joint_3, joint_4, joint_5, joint_6, joint_7

## 13.2 JSON carried in std_msgs/msg/String

The data field contains one JSON value. Command handlers expect a JSON object. Sensor serialization recursively handles dataclasses, Pydantic-like model_dump(), NumPy-like tolist(), SDK to_list(), dictionaries, lists, tuples, primitives, and finally repr().

### Hand JSON commands

| Payload | Effect |
| --- | --- |
| {"angles": [...]} | Calls set_angles. |
| {"speeds": [...]} | Calls set_speeds. |
| {"torques": [...]} | Calls set_torques. |
| {"action":"clear_faults"} | Calls the external fault-clear method. Supported only by L6 and L20. |

### Arm JSON commands

| Payload field/action | Effect |
| --- | --- |
| action: enable | Calls enable(). |
| action: disable | Calls disable(). |
| action: home | Calls home(blocking=...). |
| action: reset_error | Calls reset_error(). |
| action: emergency_stop | Calls emergency_stop(). |
| joints | Calls move_j(values, blocking=...). |
| velocities | Calls set_velocities(values). |
| accelerations | Calls set_accelerations(values). |

An unknown arm action is ignored; recognized array fields in the same object are still applied. Processing order is recognized action, joints, velocities, accelerations. The blocking field is converted with Python bool(...), so send a real JSON boolean: a nonempty string such as false evaluates as true.

# 14. API Usage

## 14.1 Publish a typed hand command

ros2 topic pub --once --wait-matching-subscriptions 1 \
  /realhand/left/hand/command sensor_msgs/msg/JointState \
  "{name: ['thumb_flex','thumb_abd','index','middle','ring','pinky'], \
    position: [50,50,50,50,50,50], \
    velocity: [60,60,60,60,60,60], \
    effort: [30,30,30,30,30,30]}"

## 14.2 Arm startup and movement

# Enable first.
ros2 topic pub --once --wait-matching-subscriptions 1 \
  /realhand/left/arm/command_json std_msgs/msg/String \
  "{data: '{\"action\":\"enable\"}'}"

# Send seven joint angles in radians.
ros2 topic pub --once --wait-matching-subscriptions 1 \
  /realhand/left/arm/joint_command sensor_msgs/msg/JointState \
  "{name: ['joint_1','joint_2','joint_3','joint_4','joint_5','joint_6','joint_7'], \
    position: [0,0,0,0,0,0,0]}"

## 14.3 Direct adapter API

The adapters are importable Python classes:

from realhand_ros2.hand import HandAdapter

hand = HandAdapter(
    model="L6",
    side="left",
    interface_name="can0",
    interface_type="socketcan",
)
try:
    hand.set_angles([50] * 6)
    hand.set_speeds([40] * 6)
    hand.set_torques([30] * 6)
    print(hand.snapshot_payload())
finally:
    hand.close()

# 15. Common Workflows

## 15.1 First hand bring-up

1. Configure can0.

2. Source ROS 2 and the workspace.

3. Launch the selected model, side, and backend.

4. List the node and topics.

## 15.2 Dual-hand operation

Launch dual_hand.launch.py, then inspect the two topic namespaces:

The provided dual launch supports a common model only. For different left/right models, start two hand.launch.py processes with distinct model, side, and interface arguments.

## 15.3 GUI hand workflow

The GUI checks for changed hand positions every 100 ms and publishes only when values differ; preset and home actions publish immediately. Auto Grab uses per-finger baseline subtraction and two-sample threshold debounce.

## 15.4 Slip detection and torque boost

The GUI computes touch magnitude change and pressure-centroid displacement. When both configured thresholds are crossed outside the cooldown window, it marks a slip. Optional torque boost increments command values for joints mapped to that finger and clamps them to 100.

# 16. Troubleshooting

## 16.1 Command fails or moves unexpected joints

Confirm model and array length.

Use the exact implementation-defined order in Section 13.

Do not assume JointState.name causes reordering; it does not.

Remember that arm command effort means acceleration.

# 17. Frequently Asked Questions (FAQ)

## Does JointState.name determine command ordering?

No. Both backend callbacks pass numeric arrays directly. Use the exact order for the selected model.

## What does JointState.effort mean?

For hand commands it is torque. For hand state it is the cached torque vector. For arm commands it is acceleration. For arm state it is the SDK torque reading.

## Can I use different models in dual_hand.launch.py?

No. That launch file has one common model argument. Launch two single-hand instances for mixed models.

## Can I change parameters at runtime?

The nodes read operational parameter values during construction and do not register a parameter-change callback.

## How do I select right-side topics?

Launch with side:=right and use /realhand/right/.... The GUI recreates its topics when the side selection changes.

## Can I disable hardware connection for graph testing?

Yes, when running the node directly, set connect_on_start:=false. Topics exist and commands are ignored with warnings, but no simulated state is published.

# 18. Appendix

## 18.1 Package metadata

| Item | Value |
| --- | --- |
| Package name | realhand_ros2 |
| Version | 0.1.0 |
| Maintainer | realhand |
| Maintainer email | realhand@todo.todo (placeholder value in source) |
| Typed package marker | src/realhand_ros2/py.typed |

## 18.2 Console entry points

| Executable | Python entry point |
| --- | --- |
| realhand_hand_node | realhand_ros2.hand.node:main |
| realhand_arm_node | realhand_ros2.arm.node:main |
| realhand_gui | realhand_ros2.gui.app:main |

## 18.3 GUI operational defaults

| Setting | Default |
| --- | --- |
| Hand position publish check | 100 ms |
| Live speed/torque debounce | 100 ms |
| Preset cycle period | 1,000 ms |

## 18.4 Useful inspection commands

ros2 pkg prefix realhand_ros2
ros2 pkg executables realhand_ros2
ros2 node list
ros2 topic list -t
ros2 topic info -v /realhand/left/hand/state
ros2 topic hz /realhand/left/hand/state
ros2 param dump /realhand_hand_node
ros2 service list
ros2 action list
ip -details -statistics link show can0

End of document